Masakhane Technologies – Master Network Build Table (Version 1)

1.  Network Infrastructure

| Device | Hostname | Purpose | Connected To | Port Used |
| --- | --- | --- | --- | --- |
| Cloud | ISP Cloud | internet | MT-RTR01 | Ethernet |
| Router (2901) | MT-RTR01 | Connects the company to the ISP | MT-EDGE-RTR01 | G0/0 ---G0/1 |
| Edge Router | MT-EDGE-RTR01 | Connects external networks to the internal enterprise network | MT-RTR01 & MT-CORE-SW01 | G0/0 & G0/1 |
| Layer 3 Switch (3560) | MT-CORE-SW01 | Inter-VLAN Routing | MT-EDGE-RTR01, MT-ACCESS-SW01,MT-ACCESS-SW02,MT-ACCESS-SW03 | Gigabit interfaces |
| Layer 2 Switch (2960) | MT-ACCESS-SW01 | Connects end devices | MT-CORE-SW01 | G0/1 |
| Layer 2 Switch (2960) | MT-ACCESS-SW02 | Connects end devices | MT-CORE-SW01 | G0/1 |
| Layer 2 Switch (2960) | MT-ACCESS-SW03 | Connects end devices | MT-CORE-SW01 | G0/1 |

2. Wireless Infrastructure

| Device | Hostname | Coverage Area | Connected To | Switch Port |
| --- | --- | --- | --- | --- |
| Access Point | MT-AP01 | HR & Management | MT-ACCESS-SW01 | Fa0/2 |
| Access Point | MT-AP02 | Central office/additional HR coverage | MT-ACCESS-SW01 | Fa0/3 |
| Access Point | MT-AP03 | IT & Reception | MT-ACCESS-SW02 | Fa0/1 |
| Access Point | MT-AP04 | Finance & Sales | MT-ACCESS-SW02 | Fa0/2 |

3. Servers

| Device | Hostname | Purpose | Connected To | Port |
| --- | --- | --- | --- | --- |
| Server | MT-SRV01 | DHCP, DNS, FTP & Central Services | MT-ACCESS-SW01 | Fa0/1 |

4.  Network Printers

| Device | Hostname | Primary Department | VLAN | Connected To | IP address |
| --- | --- | --- | --- | --- | --- |
| Printer | MT-PRN01 | HR | VLAN 30 | MT-ACCESS-SW01 | 192.168.30.20 |
| Printer | MT-PRN02 | Finance | VLAN 40 | MT-ACCESS-SW02 | 192.168.40.20 |

5. VLAN Plan

| VLAN ID | VLAN Name | Network | Default Gateway |
| --- | --- | --- | --- |
| 10 | Management | 192.168.10.0/24 | 192.168.10.1. |
| 20 | Reception | 192.168.20.0/24 | 192.168.20.1. |
| 30 | HR | 192.168.30.0/24 | 192.168.30.1. |
| 40 | Finance | 192.168.40.0/24 | 192.168.40.1. |
| 50 | Sales | 192.168.50.0/24 | 192.168.50.1. |
| 60 | IT | 192.168.60.0/24 | 192.168.60.1. |
| 70 | Server | 192.168.70.0/24 | 192.168.70.1. |
| 80 | Guest | 192.168.80.0/24 | 192.168.80.1. |

6. Wireless Networks

| SSID | Purpose | Notes |
| --- | --- | --- |
| Masakhane-Corp | Employee WI-FI | Secure corporate Wi-Fi providing access to internal resources according to VLAN membership and security policies... |
| Masakhane-Guest | Guest WI-FI | Internet access only. Isolated from all internal company resources through VLAN separation and firewall policies. |

7. End Devices Summary

| Department | Devices |
| --- | --- |
| Management | 4 Laptops |
| Reception | 2 Laptops |
| Human Resources | 6 Laptops |
| Finance | 6 Laptops |
| Sales | 10 Laptops |
| IT | 12 Laptops |
| Server | 1 Server |
| Printers | 2 Network Printers |
| Access Points | 4 Wireless Access Points |
| Core Switch | 1 |
| Access switch | 3 |
| Firewall | 1 |
| Routers | 2 |
| ISP Cloud | 1 |

8 IP Addressing Summary

| VLAN | Network | Gateway |
| --- | --- | --- |
| Management | 192.168.10.0/24 | 192.168.10.1 |
| Reception | 192.168.20.0/24 | 192.168.20.1 |
| Human Resources | 192.168.30.0/24 | 192.168.30.1 |
| Finance | 192.168.40.0/24 | 192.168.40.1 |
| Sales | 192.168.50.0/24 | 192.168.50.1 |
| IT | 192.168.60.0/24 | 192.168.60.1 |
| Servers | 192.168.70.0/24 | 192.168.70.1 |
| Guest | 192.168.80.0/24 | 192.168.80.1 |